LED Wall Wash Light

12*3W

☞ DMX MODE

3Channels  Mode

| Channel | Function | Instruction |
| --- | --- | --- |
| CH1 | R Dimmer | Red Dimmer- Dark to Bright |
| CH2 | G Dimmer | Green Dimmer- Dark to Bright |
| CH3 | B Dimmer | Blue Dimmer- Dark to Bright |

7 Channels  Mode

| Channel | Function | Instruction |
| --- | --- | --- |
| CH1 | Total Dimmer | RGBW Total Dimmer- Dark to Bright |
| CH2 | R Dimmer | Red Dimmer- Dark to Bright |
| CH3 | G Dimmer | Green Dimmer- Dark to Bright |
| CH4 | B Dimmer | Blue Dimmer- Dark to Bright |
| CH5 | Total Strobe | RGBW Total Flash Strobe- Slow to Fast |
| CH6 | Function Choice | 000~050: CH1-CH8 Control. 051~100: Different Colors output 101~150: Colors Jump Change. 151~200: Colors Gradate. 201~250: Colors Pulse Change. 251~255: Sound-Active |
| CH7 | Function/Speed | Slow to Fast (At the Colors output and sound choose the color) |

☞ KEYS INSTRUCTION

☞ DIGITAL SHOW FUNCTION FORM

| No | Show | Function | Value | Instruction |
| --- | --- | --- | --- | --- |
| 1 | A001 | DMX address | 001~512 | 7CH mode,UP,DOWN increase or reduce the value, |
| 2 | d001 | DMX address | 001~512 | 3CH mode UP,DOWN increase or reduce the value |
| 3 | r255 | Red dim | 000~255 | UP,DOWN Change brightness |
| 4 | G255 | Green dim | 000~255 | UP,DOWN Change brightness |
| 5 | B255 | Blue dim | 000~255 | UP,DOWN Change brightness |
| 6 | F255 | Strobe | 000 ~ 255 | UP,DOWN change the speed |
| 7 | CC00 | Colors choose | 00 ~ 15 | UP,DOWN Change color |
| 8 | dE00 | Colors Gradate | 00 ~ 15 | UP,DOWN change the speed |
| 9 | CP00 | Colors Pulse | 00 ~ 15 | UP,DOWN change the speed |
| 10 | SOUND | Colors | / | Sound-Active |