### AHB7016T-LY-V3

Feature：

- Support H.265+ encoding, ultra-low bitrates, compatible with H.264 surroundings;

- Support HDMI full HD output simultaneously;

- Support ONVIF; access third party;

- Support multiple cloud technology, network penetration, alarm information pushed to your phone and other functions;

- Support web, VMS, center platform management software MYEYE, provide SDK development;

- Support various mobile monitoring (iPhone, Android);

- Remote Access, With DNS (ARSP),easy to monitor remotely

- Support UTC, control signal& video stream transmit through one single coaxial

- Support XVI/AHD/TVI/CVI/CVBS/IP six signals;

- Support intelligent analysis: Human detection, face detection, Vehicle shape detection;

Specification：

|  | Model | AHB7016T-LY-V3 |
| --- | --- | --- |
| System | Main processor | NT98332 |
| Interface | Preview | 1/4/8/9/16 |
| Video | Codec | H.265+ |
|  | Monitor quality | CVBS、VGA (1080P at most)；HDMI (4K at most) |
|  | Preview ability | 16*1080N(6 in 1) |
|  | Encode ability | 16*1080N@15fps(6 in 1) |
|  | Decode ability | 16*1080N@15fps(6 in 1) |
|  | Multi-mode input | Analog：16*1080N hybrid：8*1080N（analog）+8*1080P（network） network：16*1080P；4*5M |
|  | Motion Detection | Each screen can be set to 192 (16 * 12) detection areas; multi-level sensitivity can be set (Limited local channel) |
| Audio | Coaxial audio | support |
|  | Audio compression | G.711A |
|  | Intercom | support |
| Record & playback | Record mode | manual>alarm>motion detection>timing |
|  | Local Playback | 16ch(analog input mode) |
|  | Search mode | Search by time, calendar, event, channel |
| Storage & backup | Video storage | HDD, network |
|  | Backup mode | Network, USB hard disk, USB burner, SATA burner |
| External Interface | Video input | 16ch BNC |
|  | Video output | 1ch HDMI（Can be extended to VGA*1） |
|  | Loop Vout | N/A |
|  | Audio I/O | 2（Can be extended to 6ch）/1 |
|  | Alarm I/O | 0（Can be extended to 4ch）/0（Can be extended to 1ch） |
|  | Network interface | RJ45 10M/100M self-adaption |
|  | PTZ control | 1*RS485, Support kinds of PTZ protocols |
|  | USB | 2* USB 2.0 ports |
|  | HDD | 2*SATA ports (Max 8TB/per) |
|  | Wireless | 3G, WIFI (extend via USB) |
| General | Power supply | 12V/2A |
|  | Power Consumption | <15W (without HDD) |
|  | Working environment | Temperature:0℃—+55℃，Humidity:10%—90%，Atm:86kpa—106kpa |
|  | Dimension | 239mm*85mm |
|  | Weight | 248g |

| No. | Interface Description | Function |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CN6 | 10PIN front panel socket | 1、+3.3 | 2、GND | 3、FRONT_IR | 4、FRONT_Y2 |
|  |  | 5、FRONT_Y1 | 6、FRONT_X2 | 7、FRONT_Y3 | 8、FRONT_X1 |
|  |  | 9、FRONT_ALARM | 10、FRONT_REC |  |  |
| J2 | SATA power socket |  |  |  |  |
| CN3,CN4 | SATA |  |  |  |  |
| CN5 |  |  |  |  |  |
| CN8 | USB expansion seat | 1、+5 | 2、DM | 3、DP | 4、GND |
| CN2 | MCU front panel socket | 1、+5 | 2、GND | 3、+5_STB | 4、GND |
|  |  | 5、 | 6、 | 7、SIN | 8、SOUT |
|  |  | 9、PWR_S | 10、GND |  |  |
| CN7 | FAN power socket | 1、+12 | 2、GND |  |  |
| CN13 | 485 | 1、RS485_A | 2、RS485_B |  |  |
| CN11 | Audio input |  |  |  |  |
| J1 | 433 | 1、GND | 2、SOUT | 3、SIN | 4、+3.3 |
| J3 | Switch terminal |  |  |  |  |
| CN12 | 12V input |  |  |  |  |
| CN10 | RJ45+2*USB |  |  |  |  |
| CN9 | VGA |  |  |  |  |
| J4 | HDMI |  |  |  |  |
| CN14-CN21 | BNC |  |  |  |  |
| CN1 | Audio&alarm expansion seat | 1、GND | 2、+5 | 3、 | 4、 |
|  |  | 5、 | 6、 | 7、AUDIO6 | 8、AUDIO7 |
|  |  | 9、GND | 10、GND | 11、AUDIO2 | 12、AUDIO5 |
|  |  | 13、AUDIO4 | 14、AUDIO3 | 15、AUDIO8 | 16、ALARM_IN1 |
|  |  | 17、ALARM_IN4 | 18、ALARM_IN2 | 19、ALOUT1 | 20、ALARM_IN3 |
| CN22 | Audio output seat / CVBS output seat |  |  |  |  |
| BT1 | RTC Battery seat |  |  |  |  |